RESEARCH DOCUMENT

# Literature Survey

Researcher and Author
Howida Gharib Saad El Din Selim

Research Project Version 0.2 - Validated Draft

2026-08-29

# Survey Methodology

The survey prioritizes 2021-2026 publications while retaining foundational Hamming, GDI, and FS-GDI sources. Publisher pages and DOI records are used to verify existence. Unavailable metadata are marked rather than inferred.

| Reference | Year | Circuit | Logic | Contribution | Limitation |
| --- | --- | --- | --- | --- | --- |
| Hamming | 1950 | Error-correcting codes | Coding theory | Foundational Hamming-code theory | No circuit implementation |
| Morgenshtein Fish Wagner | 2002 | Combinational gates | GDI | Introduced GDI methodology | Reduced swing and nonstandard body/source concerns motivate restoration |
| Morgenshtein Yuzhaninov Kovshilovsky Fish | 2014 | 16-bit CLA | FS-GDI | Full-swing GDI and robustness methodology | CLA rather than ECC encoder |
| Radhakrishnan Nirmalraj Karn | 2019 | Logic gates and full adder | Enhanced GDI | Enhanced-GDI restoration evaluated in LTspice | Different node and very different loads/frequencies |
| Hasan Zaman Hossain et al. | 2020 | Scalable 1-bit full adder | Hybrid full-swing GDI | Hybrid GDI full-adder implementation | Arithmetic cell rather than Hamming encoder |
| Publisher record: GDI synthesis study | 2021 | Combinational synthesis | GDI with MUX/BDD decomposition | Systematic mapping of Boolean networks | Not specific to ECC and full metadata needs subscription verification |
| Ghorbani Dolatshahi Zanjani Barekatain | 2022 | Full adder and XOR | Dynamic-GDI | Combines dynamic logic and GDI | Non-CMOS CNFET results are not directly comparable |
| Tripathi Jana Bhaumik | 2023 | SEC-DAEC and SEC-DAEC-TAEC codecs | HDL/synthesized architectures | Stronger adjacent-error correction codes | More complex codes and incompatible methodology |
| Bahrami Shiri Pesaran | 2023 | Approximate subtractors | GDI with dynamic threshold | Explores compact GDI structures | Approximate logic is unsuitable for exact ECC parity |
| Shiri et al. | 2023 | Approximate full adders | GDI/dynamic threshold | Energy-efficient GDI approximate adders | Approximation violates exact encoder requirements |
| El-Bendary El-Badry | 2024 | Hamming (7 | 4) and (11 | 72 for FS-GDI Hamming (11 | 7) |
| Albert Thiagarajan Sinthia Christopher Suganthi Arunkumar | 2024 | Boolean functions and full adder | Optimized FS-GDI | Warns that direct FS-GDI mapping is function dependent | Journal indexing/metadata quality should be checked before final submission |
| IEEE ESCI conference paper | 2024 | Hamming encoder and decoder | GDI versus CMOS | Recent GDI Hamming codec application | Insufficient accessible setup for direct quantitative comparison |
| Kim et al. | 2026 | Long Hamming encoders/decoders | Word-serial architecture | Word-serial trade-off for long codes | Targets long codewords rather than transistor-level short encoders |

Foundational work established Hamming codes, GDI, and FS-GDI. Later work repeatedly demonstrated reduced transistor count or energy in arithmetic cells, and the baseline 2024 paper applied a six-transistor FS-GDI XOR to Hamming (7,4) and (11,7). Recent ECC research also covers SEC-DED and stronger adjacent-error codes. Therefore, neither extended Hamming nor GDI alone is novel.

The defensible gap is narrower: the accessible literature does not provide a reproducible, fair, transistor-level comparison of CMOS, reconstructed FS-GDI, and selectively restored FS-GDI/CMOS extended Hamming (12,7) encoders using one predictive model, one activity pattern, explicit D1-to-P0 delay measurement, automated `.meas` extraction, and controlled VDD, temperature, load, and frequency sweeps.

The baseline paper stops at Hamming (11,7), reports no overall parity output, does not publish its netlists, and contains inconsistent delay-improvement percentages. It also treats transistor count as area. This thesis addresses those methodological weaknesses and tests two limited hybrid candidates rather than assuming that buffering improves every metric.

The resulting evidence shows a real trade-off. Hybrid-B is faster than CMOS and FS-GDI on the measured D1-to-P0 path, while FS-GDI retains the lowest simulated power and PDP. The scientific contribution is therefore a traceable characterization and a speed-oriented selective-restoration architecture, not a claim of universal superiority.