DESIGN DOCUMENT

# Proposed Hybrid Extended Hamming (12,7) Encoder

Researcher and Author
Howida Gharib Saad El Din Selim

Research Project Version 0.2 - Validated Draft

2026-08-29

# Code Architecture

The design adds P0 to Hamming (11,7) and evaluates two restoration placements.

P0 = D1 XOR ... XOR D7 XOR P1 XOR P2 XOR P4 XOR P8

# Hybrid-B Architecture Summary

Data inputs D1 through D7 feed FS-GDI XOR parity networks.

The overall parity path adds P0 for SEC-DED detection.

CMOS restoration is applied only to externally observed parity outputs.

Hybrid-B is a speed-oriented restoration compromise, not a universal energy optimum.

# Circuit-Level Alternatives

CMOS uses 12-transistor XOR cells. The reconstructed FS-GDI cell uses six transistors. Hybrid-A adds local CMOS restoration to every cell. Hybrid-B adds non-inverting restoration only at parity outputs.

# Measured Trade-off

| Architecture | Power (uW) | Delay (ps) | PDP (fJ) | Swing (V) | Transistors | Status |
| --- | --- | --- | --- | --- | --- | --- |
| CMOS | 27.0237 | 344.947 | 9.3217 | 1.2064 | 264 | Pass |
| FSGDI | 2.5502 | 429.062 | 1.0942 | 1.2135 | 132 | Pass |
| HYBRID_A | 25.8367 | 278.783 | 7.2029 | 1.2039 | 220 | Pass |
| HYBRID_B | 11.2029 | 215.846 | 2.4181 | 1.2042 | 152 | Pass |

Hybrid-B is selected for speed; FS-GDI remains the energy optimum under the tested nominal condition.