TECHNICAL DOCUMENT

# Simulation Measurement Definitions

Researcher and Author
Howida Gharib Saad El Din Selim

Research Project Version 0.2 - Validated Draft

2026-08-29

# Measurement Definitions

Average power: time average of positive power drawn from VSUP, calculated as `AVG(-V(VDD)*I(VSUP))`.

tpLH / tpHL: elapsed time from the 50% input crossing to the corresponding 50% output crossing.

Propagation delay: arithmetic mean of tpLH and tpHL.

PDP: average power multiplied by propagation delay; recalculated in Python rather than copied manually.

Energy per encoding operation: average encoder power divided by the input-word rate used by the activity pattern.

Energy per encoded bit: energy per operation divided by 11 or 12, consistently by codeword length.

Output swing: maximum measured output minus minimum measured output.

Hardware complexity: transistor count from the instantiated cell topology. It is not layout area.